ST-ADVANCED EMBEDDED SYSTEMS

TITLE: Real-Time Operator Attention Monitoring System

Final Project Report

Authors: Jula Sherene

Hardware Platform

STM32N657X0 Nucleo-N657X0  |  STEVAL-MKBOXPRO SensorTile.box PRO

April 2026

# 1. Summary

This report presents the design, implementation, and evaluation of a Real-Time Operator Attention Monitoring System developed as part of the Advanced Embedded Systems course. The system addresses a fundamental challenge in human-machine interaction: attention, an internal cognitive state, is invisible and cannot be directly measured. Instead, this project demonstrates that attention can be reliably inferred from multiple external, non-invasive signals.

The system integrates three sensing modalities: a standard webcam for face and eye detection, the STEVAL-MKBOXPRO SensorTile.box PRO for ambient noise and body motion sensing, and the NUCLEO-N657X0 with STM32N6 Neural-ART NPU for high-speed embedded video streaming. A digital signal processing pipeline cleans and fuses the multi-modal sensor data in real time, producing one of four cognitive attention states: FOCUSED, DISTRACTED, OVERLOADED, or AWAY.

The system achieves real-time classification at approximately 10 frames per second over a 1.8 Mbaud UART connection, with calibrated noise and motion thresholds derived from a 45-second per-class environmental calibration procedure. This work demonstrates that edge-embedded multi-sensor fusion is a viable, low-cost, and non-invasive approach to real-time attention monitoring with direct applications in transportation safety, workplace monitoring, and adaptive learning.

# 2. Introduction and Motivation

## 2.1 Problem Statement

Human attention is one of the most critical factors in safety-critical and productivity-sensitive environments. Inattentive truck drivers, distracted operators, and disengaged students are responsible for a significant proportion of accidents, errors, and failures in their respective domains. Despite its importance, attention is fundamentally an internal cognitive state, it cannot be directly observed or measured without invasive brain-computer interface technology.

Existing solutions either rely on expensive EEG headsets, are intrusive and uncomfortable, require significant setup time, or fail to operate in real time. There is a clear need for a non-invasive, low-cost, real-time attention monitoring system that can be deployed in practical environments.

## 2.2 Project Objectives

- Design and implement a multi-modal sensor fusion system for real-time attention monitoring

- Integrate the STEVAL-MKBOXPRO SensorTile.box PRO for acoustic and motion sensing

- Demonstrate high-speed embedded video streaming on the NUCLEO-N657X0 STM32N6 platform

- Apply digital signal processing techniques to extract clean features from raw sensor data

- Develop a calibration procedure to adapt thresholds to individual environments

- Classify attention into four meaningful states: FOCUSED, DISTRACTED, OVERLOADED, AWAY

- Provide a professional real-time GUI for demonstration and monitoring

# 3. System Architecture

## 3.1 High-Level Architecture

The system follows a three-layer architecture: sensing, processing, and classification. Three independent sensing modalities feed into a Python-based processing engine that applies DSP techniques before passing features to the fusion classification engine.

| Layer | Component | Function |
| --- | --- | --- |
| Sensing | Webcam (USB) | Face and eye detection via computer vision |
| Sensing | SensorTile.box PRO Microphone | Ambient noise level measurement |
| Sensing | SensorTile.box PRO Accelerometer | Body and desk motion measurement |
| Sensing | Nucleo N657X0 | High-speed 128x128 video frame streaming |
| Processing | Python + OpenCV | Face detection, image preprocessing |
| Processing | STDATALOG-PYSDK | SensorTile USB data acquisition |
| Processing | DSP Pipeline | RMS calculation, filtering, thresholding |
| Classification | Fusion Engine | Multi-modal attention state classification |
| Output | Tkinter GUI | Real-time visualisation and monitoring |

## 3.2 Data Flow

The data flow begins at the sensor layer where three independent streams are captured simultaneously using Python threading. The webcam stream is captured via OpenCV and processed for face detection. The SensorTile streams microphone and accelerometer data via USB using the STDATALOG-PYSDK framework with DATALOG2 firmware. Each frame captured by the webcam is simultaneously sent to the Nucleo N657X0 via UART at 1,843,200 baud for embedded video processing.

All three sensor streams converge in the fusion engine, which combines face detection status, noise level classification, and motion level classification to produce the final attention state output every 100 milliseconds.

## 3.3 Hardware Components

### 3.3.1 NUCLEO-N657X0 (STM32N6)

- Processor: Arm Cortex-M55 at 800 MHz

- Neural-ART NPU: Hardware neural network accelerator

- UART1 at 1,843,200 baud for high-speed video streaming

- Receives 128x128 grayscale frames from laptop webcam

- Sends frame confirmation back to Python host

- Firmware: Custom STM32CubeIDE project with HAL UART

### 3.3.2 STEVAL-MKBOXPRO SensorTile.box PRO

- Firmware: FP-SNS-DATALOG2 v3.2.0 (DATALOG2_Release.bin)

- Connection: USB via WinUSB driver (Zadig)

- SDK: STDATALOG-PYSDK v1.3.0

- Microphone: MP23DB01HP — ambient noise level detection

- Accelerometer: LSM6DSV16X — 3-axis body motion detection at 666 Hz

- Data format: Raw .dat files read via SensorAcquisitionThread

### 3.3.3 Webcam

- Standard USB webcam at 640x480 resolution

- OpenCV Haar Cascade face detection

- Frames resized to 128x128 grayscale for Nucleo streaming

- Face detection applied on full 640x480 frame for accuracy

# 4. Firmware and Software Implementation

## 4.1 Nucleo N657X0 Firmware (C / STM32CubeIDE)

The Nucleo firmware implements a synchronous frame reception protocol over UART1 at 1,843,200 baud. The protocol follows a five-step handshake for each frame:

| Step | Direction | Data | Description |
| --- | --- | --- | --- |
| A | Python → Nucleo | 0x55 (SYNC) | Python signals readiness to send a frame |
| B | Nucleo → Python | 0xAA (ACK) | Nucleo acknowledges and prepares to receive |
| C | Python → Nucleo | 16,384 bytes | 128x128 grayscale image payload in 512-byte chunks |
| D | Nucleo → Python | DONE:N | Nucleo confirms frame receipt with frame counter |
| E | Internal | DCache flush | Cache coherency maintained via SCB_CleanInvalidateDCache |

The firmware uses STM32CubeAI with the BlazeFace model architecture loaded onto the Neural-ART NPU. The AI initialisation is performed after UART is established, with the board broadcasting 'AI:OK' when ready for streaming. The trigger byte (0x01) from Python starts the AI inference pipeline.

## 4.2 SensorTile Python Integration

The SensorTile.box PRO is accessed via the STDATALOG-PYSDK using the HSDLink_v2 USB communication layer. The WinUSB driver installed via Zadig enables direct USB HSD communication, bypassing the proprietary BLE protocol used by the ST BLE Sensor mobile application.

Two sensors are enabled simultaneously: the MP23DB01HP microphone (sensor name: mp23db01hp_mic) and the LSM6DSV16X accelerometer (sensor name: lsm6dsv16x_acc). Each sensor runs in a dedicated SensorAcquisitionThread that writes raw binary data to .dat files. Python reads the last 2000 bytes of each file to compute real-time feature values.

## 4.3 Digital Signal Processing Pipeline

Four DSP techniques are applied to extract clean, meaningful features from raw sensor data:

| DSP Technique | Applied To | Purpose |
| --- | --- | --- |
| RMS Calculation | Microphone audio samples | Convert raw int16 audio to a single loudness amplitude value |
| Moving Average | All sensor streams | Smooth rapid fluctuations and reduce measurement noise |
| Calibrated Thresholding | Mic RMS and accelerometer magnitude | Classify continuous values into discrete states |
| Chunked Sliding Window | Last 2000 mic bytes / 600 acc bytes | Ensure recency bias — only recent data influences decision |

The microphone RMS is computed as: RMS = sqrt(mean(samples^2)) where samples are the last 1000 int16 audio values. The accelerometer motion magnitude is computed as: motion = mean(abs(int16_samples)) * 0.000488, where 0.000488 g/LSB is the sensitivity of the LSM6DSV16X at the 2g full-scale setting.

## 4.4 Calibration Procedure

A 45-second per-class calibration procedure establishes environment-specific thresholds. The user performs three sequential phases: silent (no noise), normal (talking), and loud (clapping/shouting). The system records RMS averages for each phase and computes threshold midpoints:

| Calibration Phase | Duration | RMS Average | Threshold Derived |
| --- | --- | --- | --- |
| Silent | 45 seconds | 1,162 | FOCUSED threshold < 1,418 |
| Normal speech | 45 seconds | 1,674 | DISTRACTED threshold < 1,770 |
| Loud noise | 45 seconds | 1,865 | OVERLOADED threshold >= 1,770 |

These calibrated thresholds replace fixed values and adapt the system to the specific acoustic environment of deployment. A similar calibration approach was applied to the accelerometer motion signal, establishing STILL (<0.4g), MOVING (<1.0g), and ACTIVE (>=1.0g) classifications.

## 4.5 Fusion Engine

The fusion engine combines face detection status, noise level, and motion level into a single attention state classification using a priority-ordered rule set:

| Priority | Condition | Attention State | Colour |
| --- | --- | --- | --- |
| 1 (Highest) | Face not detected | AWAY | Yellow #FFFF00 |
| 2 | Noise LOUD or Motion ACTIVE | OVERLOADED | Red #FF3C6E |
| 3 | Noise QUIET and Motion STILL | FOCUSED | Cyan #00FFE0 |
| 4 (Default) | Any other combination | DISTRACTED | Orange #FF8C00 |

When the SensorTile is not connected, the system falls back to 'FACE DETECTED' as an intermediate state, indicating visual attention is present but environmental context is unavailable. This graceful degradation ensures the system remains functional even with partial sensor availability.

# 5. Graphical User Interface

The GUI is implemented using Python Tkinter with a dark-themed professional design. It provides real-time monitoring of all system components and sensor states simultaneously.

| GUI Panel | Contents |
| --- | --- |
| Main Video Feed (640x480) | Live webcam with overlaid focus state, AI result, FPS, noise and motion labels |
| STM32 Preview (192x192) | 128x128 grayscale frame sent to Nucleo, upscaled for visibility |
| System Stats Panel | FPS counter, Face AI result, Noise state, Motion state, Focus State (colour-coded) |
| Control Panel | Nucleo COM port selector, camera index, Connect Nucleo button, Connect SensorTile button, Start Stream button |
| Serial Log | Real-time UART communication log with timestamps |

The GUI uses a producer-consumer pattern with a thread-safe queue for camera frames. The stream loop runs in a background thread that sends frames to Nucleo and reads sensor data, while the main GUI thread polls the queue every 30 milliseconds to update the display without blocking the Tkinter event loop.

# 6. System Flowchart

## 6.1 Startup Sequence

The system follows a defined startup sequence to ensure all hardware components are initialized before streaming begins:

- Power on SensorTile.box PRO with DATALOG2 firmware

- Installed WinUSB driver via Zadig for USB HSD communication

- Launch Python GUI application

- Click CONNECT SENSORTILE: enables microphone and accelerometer, starts acquisition threads

- Power on Nucleo N657X0 with custom UART streaming firmware

- Select Nucleo COM port and click CONNECT NUCLEO

- Python sends trigger byte 0x01: Nucleo initialises AI and broadcasts AI:OK

- Click START STREAM: webcam opens, frame streaming begins

## 6.2 Main Processing Loop

Each iteration of the main processing loop performs the following steps:

- Capture 640x480 BGR frame from webcam

- Run OpenCV Haar Cascade face detection on full frame

- Resize frame to 128x128 grayscale for Nucleo streaming

- Send SYNC byte (0x55) to Nucleo and wait for ACK (0xAA)

- Transmit 16,384-byte image payload in 512-byte chunks

- Read DONE confirmation from Nucleo

- Read latest 2000 bytes from microphone .dat file, compute RMS

- Read latest 600 bytes from accelerometer .dat file, compute motion magnitude

- Apply calibrated thresholds to classify noise and motion states

- Run fusion engine to determine final attention state

- Push results to GUI queue, update display, overlays and stats panel

# 7. Results and Performance

## 7.1 Streaming Performance

| Metric | Value |
| --- | --- |
| UART Baud Rate | 1,843,200 bps |
| Frame Size | 16,384 bytes (128x128 grayscale) |
| Achieved Frame Rate | ~10 FPS |
| ACK Latency | <100 ms per frame |
| Sensor Update Rate | 200 ms (5 Hz) |
| Focus State Update Rate | Every frame (~100 ms) |
| Calibration Duration | 135 seconds (3 x 45s phases) |

## 7.2 Attention State Classification Results

| Attention State | Trigger Conditions | Observed Behaviour |
| --- | --- | --- |
| FOCUSED | Face visible + Noise < 1418 RMS + Motion < 0.4g | Correctly triggered in quiet, still environment with face present |
| DISTRACTED | Face visible + Moderate noise or motion | Correctly triggered when talking or minor movement detected |
| OVERLOADED | Noise >= 1770 RMS or Motion >= 1.0g | Correctly triggered by clapping, loud speech, or vigorous movement |
| AWAY | No face detected by OpenCV | Correctly triggered when stepping away or covering camera |

## 7.3 DSP Signal Processing Effectiveness

The RMS-based noise classification proved effective for distinguishing environmental acoustic contexts. Calibrated thresholds adapted well to the specific room acoustics, with clear separation between silence (avg 1,162 RMS), normal speech (avg 1,674 RMS), and loud noise (avg 1,865 RMS) conditions. The accelerometer motion classification correctly distinguished between stationary (0.34-0.40g baseline gravity), slow movement (0.4-1.0g), and vigorous shaking (>1.0g).

# 8. Limitations and Prototype Disclaimer

## 8.1 System Limitations and Prototype Scope:

It is important to note that the classification results produced by this system represent a best-effort estimation based on indirect external signals, not a ground-truth measurement of internal cognitive attention. The system is a functional prototype demonstrating the feasibility of multi-modal sensor fusion for attention monitoring, rather than a clinically validated or production-ready solution.

## 8.2 Known Limitations:

Several factors may affect classification accuracy in practice. Webcam-based face and eye detection is sensitive to lighting conditions, camera angle, and face orientation. The microphone thresholds were calibrated for a specific acoustic environment and may require recalibration in different settings. The accelerometer motion signal captures desk and device vibration rather than direct body movement, which may not always correlate with attentional state. Additionally, a person can be physically still and visually present while mentally disengaged, a limitation inherent to any external signal-based attention monitoring approach.

## 8.3 Future Research Directions:

Future work would address these limitations through several enhancements. Training a supervised machine learning classifier on labelled attention data would replace the current rule-based fusion engine with a data-driven model capable of learning individual attention patterns. Integration of physiological signals such as heart rate variability or electroencephalography would provide direct measures of cognitive load. Full gaze tracking hardware would replace the current head pose proxy with precise screen region attention mapping. Longitudinal data collection would enable personalized threshold adaptation and attention pattern analysis over time.

A compelling extension of this architecture can be integrated into smart eyewear since the future is wearable device. Mounting an inward-facing eye camera, microphone, and IMU directly on a glasses frame would transform the system from a desk-based monitor into a fully wearable, context-independent attention tracker. The STM32N6 Neural-ART NPU, already used in this project for embedded video streaming; is well-suited for this application due to its ultra-low power consumption and on-chip AI inference capability. Combined with an outward-facing camera for scene understanding, smart glasses could determine not only whether the wearer is attentive, but what they are attentive to a significant advancement over current desk-based approaches.

# 9. Industry Applications

The architecture demonstrated in this project maps directly to several high-value industry applications:

| Industry | Application | Regulatory Driver |
| --- | --- | --- |
| Automotive / Transportation | Driver Monitoring Systems for trucks, buses, trains | EU mandates DMS in all new vehicles from 2024 |
| Aviation | Pilot attention monitoring during autopilot phases | EASA fatigue management regulations |
| Industrial Safety | Heavy machinery operator alertness detection | HSE workplace safety legislation |
| Education | Student engagement monitoring in e-learning platforms | Adaptive learning market growth |
| Healthcare | ADHD monitoring, cognitive rehabilitation assessment | Clinical cognitive assessment standards |
| Defence | Drone operator and mission control fatigue detection | NATO operational readiness standards |
| Consumer Productivity | Smart desk attention coaching applications | Growing personal productivity market |

Of commercial relevance is the automotive driver monitoring systems market. The EU General Safety Regulation 2019/2144 mandated that all new vehicle types sold in Europe from July 2024 must include Driver Monitoring Systems capable of detecting driver drowsiness and distraction. This represents a significant commercial opportunity directly aligned with the multi-modal sensor fusion approach demonstrated in this project.

# 10. Challenges and Solutions

| Challenge | Solution Implemented |
| --- | --- |
| SensorTile USB driver compatibility | Installed WinUSB driver via Zadig for HSD protocol access |
| STDATALOG-PYSDK submodule installation | Cloned repository with --recursive flag via Git to include .whl packages |
| Python 3.14 float formatting (printf_float) | Used integer arithmetic with /1000.0 scaling to avoid linker flag dependency |
| BlazeFace NPU inference instability | Moved face detection to OpenCV Haar Cascade on host; Nucleo retained for video streaming |
| UART communication corruption from debug messages | Removed all UART_SendString calls from main loop except ACK and DONE |
| Noise threshold sensitivity | Implemented 45-second per-class calibration to adapt to specific environment |
| ThreadX USBX CDC USB issues on SensorTile | Used DATALOG2 firmware with STDATALOG-PYSDK instead of custom firmware |
| ACK timeout in Python | Increased ACK timeout to 3.0 seconds and added input buffer flush before sync byte |

# 11. Future Work

## 11.1 Trained AI Classifier on Nucleo NPU

The most significant planned enhancement is the deployment of a trained attention classification model onto the STM32N6 Neural-ART NPU. The approach would involve collecting labelled sensor data across all four attention classes, training a Random Forest or lightweight neural network classifier in Python using scikit-learn, converting the model to TensorFlow Lite format, importing it into STM32Cube.AI, and deploying it on the Nucleo NPU for on-device inference. This would transform the rule-based fusion engine into a genuine trained Edge AI classifier.

## 11.2 Physiological Signal Integration

Future iterations could integrate heart rate variability (HRV) sensing via an optical PPG sensor such as the MAX30102, or EEG-based cognitive load measurement via a consumer-grade headset. These physiological signals provide a more direct window into internal cognitive state and would significantly improve classification accuracy.

## 11.3 Gaze Tracking

Replacing simple face detection with full gaze tracking would provide a significantly more precise measure of visual attention. Rather than simply detecting face presence, the system could determine whether the user's gaze is directed at the work area, a phone, or another distraction source.

## 11.4 Longitudinal Monitoring

Extending the system to log attention states over time would enable longitudinal analysis of attention patterns, identification of peak focus periods, and personalized productivity recommendations. This data could also serve as a rich training data set for future AI model improvements.

# 12. Conclusion

This project has successfully demonstrated that internal human attention can be meaningfully inferred from external, observable signals in real time. By combining computer vision-based face detection, acoustic environment monitoring, and motion sensing in a multi-modal sensor fusion architecture running on embedded hardware, the system reliably classifies operator attention into four actionable states.

The project makes several contributions to the field of embedded systems and human-machine interaction. It demonstrates practical integration of the STEVAL-MKBOXPRO SensorTile.box PRO with a Python host using the STDATALOG-PYSDK framework. It demonstrates high-speed video streaming over UART at 1.8 Mbaud on the STM32N6 platform with Neural-ART NPU. It implements a complete DSP pipeline including RMS calculation, calibrated thresholding, and multi-modal signal fusion. And it provides professional real-time monitoring GUI with live sensor state visualization.

The answer to the research question ‘Can internal attention be measured through external signals?'is affirmatively yes: imperfectly, but usefully. In safety-critical applications, an imperfect real-time early warning is infinitely more valuable than no warning at all. This project establishes the foundational architecture for a practical, non-invasive, embedded attention monitoring system with direct applicability across transportation, industrial, educational, and healthcare domains.

# SAMPLE RESULTS (FUTURE ACCURACY FOCUSED):

## Attention State Classification Truth Table

| Face Detected | Eyes Visible | Head Centered | Noise Level | Motion Level | Attention State |
| --- | --- | --- | --- | --- | --- |
| ❌ No | — | — | — | — | 🟡 AWAY |
| ✅ Yes | ❌ No | — | — | — | 🟠 DISTRACTED |
| ✅ Yes | ✅ Yes | ❌ No (turned) | — | — | 🟠 DISTRACTED |
| ✅ Yes | ✅ Yes | ✅ Yes | LOUD ≥1770 | — | 🔴 OVERLOADED |
| ✅ Yes | ✅ Yes | ✅ Yes | — | ACTIVE ≥1.0g | 🔴 OVERLOADED |
| ✅ Yes | ✅ Yes | ✅ Yes | QUIET <1418 | STILL <0.4g | 🟢 FOCUSED |
| ✅ Yes | ✅ Yes | ✅ Yes | MODERATE | STILL | 🟠 DISTRACTED |
| ✅ Yes | ✅ Yes | ✅ Yes | QUIET | MOVING | 🟠 DISTRACTED |
| ✅ Yes | ✅ Yes | ✅ Yes | MODERATE | MOVING | 🟠 DISTRACTED |